# CHAPTER 10	DOS APPLICATION INTERFACE§§

## 1.10	API LIBRARIES

A C language libraryXE "C language library"§XE "library"§XE "Source code"§ of interface functions is provided.  This library was written using Microsoft C 7.0 (C language definitions are included for conversion to Borland Turbo C).  The following files are included:

ARIA_S.LIB	Library for small model compiler

ARIA_C.LIB	Library for compact model compiler

ARIA_M.LIB	Library for medium model compiler

ARIA_L.LIB	Library for large model compiler

ARIA.C	Source code file (Aria command functions)

ARIAMIDI.C	Source code file (Aria Synthesizer functions)

ARIADSP.C	Source code file (DSP application functions)

AIM.ASM	Source code file (Aria Interrupt Manager)

PCM_ISR.ASM	Source code file (Aria PCM audio ISR)

ARIA.H	Include file containing Aria definitions

ARIAMIDI.H	Include file for ARIAMIDI.C

PITCHES.H	Include file for ARIAMIDI.C

ARIADSP.H	Include file for ARIADSP.C

ARIA.INC	Include file for AIM.ASM and PCM_ISR.ASM

A C language function is provided for each low-level Aria command.  In addition, a MIDImessage function is included to translate and execute MIDI events directly.

The following chapter will detail the purpose, input arguments, and return values of each supplied API function.  The functions are arranged alphabetically.

NOTE:  Before calling any Aria PCM Audio or Aria Synthesizer function, the SystemInit function must be called to select Aria mode.  When an application is finished, SystemInit should be called again to return the board to default mode.

## 1.20	API FUNCTIONS

| Initialization Functions |  |
| --- | --- |
| SystemInit () | Initialize to Aria mode or Sound Blaster emulation |
| SelectDSPApp () | Load or remove an Aria DSP application |
| GetAriaCaps () | Return Aria system information and capabilities |
| GetAriaEnv () | Return Aria DOS environment variables |
| GetVersion () | Return Aria system ROM version number |
| GetDSPVersion () | Return Aria DSP version number |
| GetInterrupt () | Return Aria interrupt (IRQ) number |
| GetDMAChannel () | Return Aria DMA channel |
| SetPlaybackMode () | Set Aria Synthesizer playback mode |
| SetListenerMode () | Switch Aria Listener on or off |
| SetReverbMode () | Select reverb effect mode |
| SetReverbLevel () | Set master reverb level |
| SetReverbWave () | Set the digital audio reverb level |
| SetReverbChannel () | Set the reverb level for a Wave Parameter Table |
| SetQSoundMode () | Switch QSound Virtual Audio on or off |

| PCM Audio Functions |  |
| --- | --- |
| InstallAriaInt () | Install Aria PCM interrupt handler |
| RemoveAriaInt () | Remove Aria PCM interrupt handler |
| StartPlayback () | Start audio playback channel |
| StopPlayback () | Stop audio playback channel |
| StartRecording () | Start audio recording |
| StopRecording () | Stop audio recording |
| SetPlaybackVolume () | Set playback master volume |
| SetRecordingVolume () | Set recording level |
| SetAudioFormat () | Set audio format -- data bits, sample rate |
| SetChannelVolume () | Set playback channel volume |
| SetChannelPan () | Set playback channel stereo pan |
| GetAudioPosition () | Return playback or recording file position |
| GetBufferPosition () | Return playback or recording buffer position |
| SetAudioTransfer () | Specify end-of-packet transfer |
| SetPacketSize () | Set audio transfer packet size |

| Aria Synthesizer Functions (High-Level) |  |  |
| --- | --- | --- |
| InitPatchBank () | Initialize Aria instrument patch bank |  |
| MIDImessage () | Process standard MIDI event |  |
| SetSynthVolume () | Set Aria Syntheizer output volume |  |
| SetOperatorCount () | Set Aria Synthesizer operator count |  |

| Aria Synthesizer Functions (Low-Level) |  |  |
| --- | --- | --- |
| OperatorOn () | Turn on specific synthesis operator |  |
| OperatorOff () | Turn off specific synthesis operator |  |
| OperatorCancel () | Cancel specific synthesis operator |  |
| SetOperatorVolume () | Set synthesis operator amplitude (volume) |  |
| SetOperatorPan () | Set synthesis operator stereo pan |  |
| SetOperatorPitch () | Set synthesis operator frequency (pitch) |  |
| LFOactivate () | Turn on low frequency oscillator (LFO) |  |
| LFOdeactivate () | Turn off low frequency oscillator (LFO) |  |
| SetLFOdepth () | Set depth control for LFO |  |
| SetLFOrate () | Set frequency control for LFO |  |
| SetWaveParameter () | Initialize specific wave parameter table |  |
| TableModify () | Modify individual synthesis table parameter |  |

| Mixer Functions |  |  |
| --- | --- | --- |
| SetMasterVolume () | Set Aria master volume |  |
| SetMixerConfig () | Set mixer configuration - recording source, level, etc. |  |
| SetAuxVolume () | Set Auxiliary (CD-ROM audio) volume |  |
| SetExtVolume () | Set External (Microphone) volume |  |
| SelectADC () | Turn ADC on or off |  |

| Mixer Functions (SC18075 mixer only) |  |  |
| --- | --- | --- |
| SetMonitorMode () | Set input channel monitor mode |  |
| SetMasterMode () | Set output channel monitor mode |  |
| SetMixerVolume () | Set mixer channel volume |  |
| SetToneControl () | Set tone controls (treble and bass) |  |

| Miscellaneous Functions |  |  |
| --- | --- | --- |
| GetDSPUsage () | Return DSP resource utilization percentage |  |
| putMem16 () | Write a word to DSP RAM |  |
| getMem16 () | Read a word from DSP RAM |  |
| sendDSP () | Send a DSP command word |  |

GetAriaCapsXE "DOS API Functions:GetAriaCaps"§

PURPOSE	Use GetAriaCaps to obtain information regarding Aria system capabilities and specific board level features.

SYNTAX	void GetAriaCaps ( unsigned char far *cptr );

cptr	Far pointer to an ARIACAPS structure.

RETURNS	The ARIACAPS structure will be filled with information regarding the Aria system.

EXAMPLE	Determine the ROM size of the installed Aria board.

ARIACAPS ac;

GetAriaCaps ((unsigned char far *) &ac);
printf ("\nThe ROM size is %d Kwords", ac.ROMsize);

NOTES	The following is the ARIACAPS structure (defined in ARIA.H) and a description of each field:

typedef struct ariacaps_tag
	{
	UINT	version;	/* Aria ROM version number */
	UINT	ROMsize;	/* ROM size (in Kbytes):  64, 128, 256, 512,
				1024,2048, 4096, 8192 or 16384 */
	UINT	ROMwidth;	/* ROM wave table bit resolution: 8 or 16 */
	UINT	mixerType;	/* Mixer type:

0 = no mixer (for input channels)
					1 = discrete analog mixer
					4 = SC18075 mixer chip */
	UINT	RAMsize;	/* RAM size in Kwords:  8, 16, 32, 40, or 64 */
	UINT	ADCbits;	/* ADC bit resolution:  8, 10, 12 or 16 */
	BOOL	phoneInterface;	/* Phone interface present?  0=no, 1=yes */
	WORD	DSPapps;	/* DSP applications available (if bit set):
					BIT		APPLICATION
					 0			Reserved
					 1			Sound Blaster emulation
					 2			Aria PCM digital audio
					 3			Aria Synthesizer
					 4			Telephone interface
					 5			Aria Listener speech recognition
					 6			Unused
					 7			Reverb effects
					 8			QSound Virtual Audio */
	} ARIACAPS;

GetAriaEnvXE "DOS API Functions:GetAriaEnv"§

PURPOSE	Use GetAriaEnv to determine if an ARIA DOS environment variable has been defined and what parameters have been set.

SYNTAX	int GetAriaEnv (	unsigned far *port, unsigned far *irq, 
	unsigned far *dma, unsigned far *ver );

port	Pointer to integer variable that will receive Aria base address port (280h, 290h, 2A0h, or 2B0h).

irq	Pointer to integer variable that will receive Aria interrupt request number (IRQ 10, 11, or 12).

dma	Pointer to integer variable that will receive Aria DMA channel (5 or 6).

ver	Pointer to integer variable that will receive Aria system version (should be a 1).

RETURNS	GetAriaEnv returns -1 if the environment variables were not defined -- the function parameters will be set to current system defaults.  0 is returned if the environment variables are defined correctly.

SEE ALSO	GetDMAChannel, GetInterrupt

EXAMPLE	Determine the DOS environment variable for the Aria base port address.

unsigned port, irq, dma, ver;

if (GetAriaEnv ((unsigned far *) &port, (unsigned far *) &irq, 
		    (unsigned far *) &dma, (unsigned far *) &ver) == 0)
	printf ("\nAria base address port = %d", port);
else
	printf ("\nAria environment variables not set");

GetAudioPositionXE "DOS API Functions:GetAudioPosition"§

PURPOSE	Use GetAudioPosition to obtain the number of bytes remaining in a currently playing or recording digital audio sample.  By subtracting this number from the size of the sample, the offset from the beginning may be determined.

SYNTAX	unsigned long GetAudioPosition ( unsigned ch );

ch		Digital audio channel (0 to 3)

RETURNS	GetAudioPosition returns the number of bytes from the end of the selected audio sample.  0xFFFFFFFF is returned if the audio channel parameter is out of range.

SEE ALSO	GetBufferPosition

EXAMPLE	Print a running counter on the screen showing the offset in bytes from the beginning of an active audio channel.

unsigned long pos;
unsigned ch=0;
unsigned long sampsize;	/* Set to sample size in bytes */

/* After starting digital audio channel 0 playing */

while ((pos = GetAudioPosition(ch)) != 0L)
	printf ("\rOffset = %ld", sampsize - pos);

GetBufferPositionXE "DOS API Functions:GetBufferPosition"§

PURPOSE	Use GetBufferPosition to obtain the number of bytes remaining in a currently playing or recording digital audio buffer.  By subtracting this number from the size of the buffer, the offset from the beginning may be determined.

SYNTAX	unsigned long GetBufferPosition ( unsigned ch );

ch		Digital audio channel (0 to 3)

RETURNS	GetBufferPosition returns the number of bytes from the end of the selected audio channel buffer.  0xFFFFFFFF is returned if the audio channel parameter is out of range.

SEE ALSO	GetAudioPosition

EXAMPLE	Print a running counter on the screen showing the offset in bytes from the beginning of an active audio channel.

unsigned long pos;
unsigned ch=0;
unsigned long bufsize;	/* Set to buffer size in bytes */

/* After starting digital audio channel 0 playing */

while (GetAudioPosition(ch) != 0L)
	{
	pos = GetBufferPosition(ch);
	printf ("\rOffset = %ld", bufsize - pos);
	}

GetDMAChannelXE "DOS API Functions:GetDMAChannel"§XE "DMA"§

PURPOSE	Use GetDMAChannel to determine the hardware jumper setting of the DMA channel on the Aria board.  Global variable dmachan will be set to the channel value for use with other library functions.  This function should be called prior to installing a DMA-driven interrupt service routine for digital audio.

SYNTAX	int GetDMAChannel ( void );

RETURNS	The GetDMAChannel function returns -1 if unable to determine the DMA channel.  Otherwise, the DMA channel number is returned.

EXAMPLE	Print the Aria DMA channel value on the screen.

printf ("DMA channel = %d", GetDMAChannel());

GetDSPUsageXE "DOS API Functions:GetDSPUsage"§XE "DSP"§

PURPOSE	Use GetDSPUsage to determine the current maximum DSP resource utilization.

SYNTAX	unsigned GetDSPUsage ( void );

RETURNS	GetDSPUsage returns a percentage value from 0 to 100.  The Aria DSP can only load applications up to the 100 percent limit.

EXAMPLE	Determine if a new DSP application can be successfully loaded by the Aria DSP.

unsigned newAppUsage = 40;  /* New DSP application usage percentage */

if ((newAppUsage + GetDSPUsage ()) > 100)
	printf ("\nCannot load DSP application");

GetInterruptXE "DOS API Functions:GetInterrupt"§XE "Interrupts"§

PURPOSE	Use GetInterrupt to determine the hardware jumper setting of the IRQ select on the Aria board.  Global variable dspint will be set to the IRQ value for use in other library functions.  This function should be called prior to installing an interrupt service routine for playing and recording digital audio.

SYNTAX	int GetInterrupt ( void );

RETURNS	The GetInterrupt function returns -1 if the system is not in PCM Audio and Aria Synthesizer mode or -2 if unable to determine the IRQ.  Otherwise, 10, 11 or 12 is returned corresponding to the IRQ level.

SEE ALSO	InstallAriaInt, RemoveAriaInt

EXAMPLE	Print the Aria IRQ value on the screen.

SystemInit (ARIA_SYNTH);
printf ("IRQ = %d", GetInterrupt());

getMem16XE "DOS API Functions:getMem16"§XE "RAM Locations"§

PURPOSE	Use getMem16 to read a word value from DSP RAM.  This can be used to retrieve status information and read data from the PCM audio buffer.  A list of defined DSP RAM address locations can be found in Section 3.2.2, Memory Layout.

SYNTAX	unsigned getMem16 ( unsigned addr );

addr	The DSP RAM address to read.

RETURNS	getMem16 returns the data word found at the specified DSP RAM address.

SEE ALSO	putMem16

EXAMPLE	Retrieve the DSP part number code.

unsigned DSPcode;

DSPcode = getMem16 (0x60C3);
switch (DSPcode)
	{
	case 0:
		printf ("\nSC18025 DSP");
		break;
	case 1:
		printf ("\nSC18026 DSP");
		break;
	default:	
		printf ("\nUnknown DSP part code: %d", DSPcode);
	}

GetDSPVersionXE "DOS API Functions:GetDSPVersion"§XE "DSP Version"§

PURPOSE	Use GetDSPVersion to return the Aria DSP version.  Global variable DSPversion will be set when this function is called.  Library function SystemInit calls GetDSPVersion if the system has initialized properly.

SYNTAX	unsigned GetDSPVersion ( void );

RETURNS	GetDSPVersion returns the DSP major version number in the most significant byte and the minor version number in the least significant byte.  If 0 is returned, the DSP version could not be determined.  If 0xFFFF is returned, the DSP is not responding.

SEE ALSO	GetVersion

EXAMPLE	Print the Aria DSP version.

unsigned DSPver;

DSPver = GetDSPVersion ();
switch (DSPver)
	{
	case 0:
		printf ("\nCannot determine DSP version number");
		break;
	case 0xFFFF:
		printf ("\nAria DSP not responding");
		break;
	default:
		printf ("\nAria DSP version number = %d.%02d",	DSPver >> 8, 
			DSPver & 0xFF);
	}

GetVersionXE "DOS API Functions:GetVersion"§XE "ROM Version"§

PURPOSE	Use GetVersion to determine the Aria system ROM version number.  Global variable version will be set when this function is called.  Library function SystemInit calls GetVersion if the system has initialized properly.

SYNTAX	unsigned GetVersion ( void );

RETURNS	The GetVersion function returns the ROM major version number in the most significant byte and the minor version number in the least significant byte.  If 0 is returned, the ROM version could not be determined.  0xFFFF is returned if the Aria board is not responding.

SEE ALSO	GetDSPVersion

EXAMPLE	Print the Aria system ROM version number.

unsigned ROMver;

ROMver = GetVersion ();
switch (ROMver)
	{
	case 0:
		printf ("\nCannot determine ROM version number");
		break;
	case 0xFFFF:
		printf ("\nAria DSP not responding");
		break;
	default:
		printf ("\nAria ROM version number = %d.%02d",	ROMver >> 8, 
			ROMver & 0xFF);
	}

InitPatchBankXE "DOS API Functions:InitPatchBank"§

PURPOSE	Use InitPatchBank to set up the Aria system prior to playing synthesized music.  The function will initialize the system according to the specified patch bank and mode.  Either version of Aria patch bank formats may be used.

SYNTAX	int InitPatchBank ( void far *bankptr, unsigned mode );

bankptr	Far pointer to patch bank loaded into memory (see ARIAMIDI.H for bankstruct definition)

mode	0	For no synthesis operators
1	For 20 synthesis operators
2	For 32 synthesis operators
			or
High order byte can contain number of operators to allow
(1 to 32)

RETURNS	The InitPatchBank function returns 1 if the patch bank is an illegal format or version, 2 if the mode parameter is out of range, -1 if the DSP is not responding or -2 if the selected mode is not currently available.  Otherwise, 0 is returned if no error.

SEE ALSO	MIDImessage

EXAMPLE	Load and initialize the patch bank file, GENMIDI.BNK.

FILE stream;
unsigned char huge *bufptr; /* allow size > 64K */
unsigned long banksize, i;

if ((stream = fopen ("GENMIDI.BNK", "rb")) == NULL)
	{
	printf ("ERROR - file not found\n");
	exit (1);
	}
banksize = filelength (fileno (stream));
if ((bufptr = halloc (banksize / 2L + 1L, 2)) == NULL)
	{
	printf ("ERROR - out of memory\n");
	exit (1);
	}
i = 0L;	/* read in patch data 1K at a time */
while (fread (bufptr+i, 1, 1024, stream) == 1024)
	i += 1024L;
fclose (stream);

InitPatchBank ((void far *) bufptr, 2);

/* bufptr must not be deallocated until the end of the program or until synthesis is no longer required */

InstallAriaIntXE "DOS API Functions:InstallAriaInt"§XE "Interrupts"§

PURPOSE	Use InstallAriaInt to install an interrupt service routine capable of handling record and playback functions.  GetInterrupt should be called prior to installing the interrupt handler to detect the IRQ setting.  Once InstallAriaInt has been called, all digital recording and playback functions may be used.

SYNTAX	void InstallAriaInt ( void );

SEE ALSO	RemoveAriaInt

EXAMPLE	Install interrupt handler and play a preloaded sound that is 16 Kbytes in length.

unsigned char far *sndbuffer;   /* pointer to sound */
int ch=0;

SystemInit (ARIA_SYNTH);
if (GetInterrupt() > 0)
	{
	InstallAriaInt();
	SetAudioFormat (SR22KHZ | MONO | EIGHTBIT);
	StartPlayback (ch, sndbuffer, 16384L, 16384L);

/* wait until finished */
	while (GetAudioPosition (ch) != 0L) 
		;
	RemoveAriaInt();
	}

LFOactivateXE "DOS API Functions:LFOactivate"§

PURPOSE	Use LFOactivate to turn on a low frequency oscillator associated with synthesis operator(s).  This can be done prior to turning on a note that uses an LFO.  The wave parameter table (see SetWaveParameter) defines the LFO number and depth for an operator.

SYNTAX	int LFOactivate ( unsigned LFOnum, unsigned type, unsigned pitch );

LFOnum	The LFO generator number (0 to 15)

type	The LFO shape:
		0 = Ramp
		1 = Reverse Ramp
		2 = Triangle
		3 = Square
		4 = Pulse (25% duty cycle)
		5 = Staircase
		6 = Sine Wave

pitch	The LFO frequency value (xxyy in hex):
		xx = LFO rate (int. portion)  00h to 3Fh
		yy = LFO rate (frac. portion)  00h to FFh

LFO rate is computed using the following equations:

xx = (int)

Note:  Frequency is in hertz.

RETURNS	LFOactivate returns 1 if the LFO number parameter is out of range, 2 if the type parameter is out of range, or 3 if the pitch value is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	LFOdeactivate, SetWaveParameter

EXAMPLE	Turn on LFO generator number 3 as a sine wave with a frequency of 45.0 Hz.

float freq = 45.0;
unsigned LFOint, LFOfrac;

LFOint = (unsigned) (freq / 172.3);
LFOfrac = (unsigned) (((freq / 172.3) - LFOint) * 512.0);
LFOactivate (3, 6, (LFOint << 8) | LFOfrac);

LFOdeactivateXE "DOS API Functions:LFOdeactivate"§

PURPOSE	Use LFOdeactivate to turn off an active low frequency oscillation generator.

SYNTAX	int LFOdeactivate ( unsigned LFOnum );

LFOnum	The LFO generator number (0 to 15)

RETURNS	LFOdeactivate returns 1 if the LFO number is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	LFOactivate

EXAMPLE	Turn off LFO generator number 3.

LFOdeactivate (3);

XE "Low Frequency Oscillator" \r "LFO3"§

MIDImessageXE "DOS API Functions:MIDImessage"§XE "MIDI"§

PURPOSE	Use MIDImessage to send a MIDI message event to the Aria board.  All MIDI channels 1 through 16 will process messages, with MIDI channel 10 processing key-mapped percussion.  InitPatchBank must be called prior to sending any MIDI messages.

SYNTAX	int MIDImessage (	unsigned char cmd, unsigned char dat1, 
	unsigned char dat2 );

cmd	The MIDI command code (channels 1 to 10):
	0x80 to 0x89	Note Off
	0x90 to 0x99	Note On
	0xB0 to 0xB2	Control Change
	0xC0 to 0xC9	Program Change
	0xE0 to 0xE9	Pitch Bend

dat1	First MIDI data byte as defined by the MIDI specification.  
Defined Control Change parameters are:
	1		Modulation (if defined by patch)
	6		Data Entry
	7		Channel volume
	10		Channel pan
	64		Hold1 (Sustain)
	91		Effect1 depth (Reverb level)
	100,101	RPN LSB, MSB
	120		All sounds off 
	121		Reset controllers
	123-127	All notes off

Defined Registered Parameter Numbers (RPN):
	0		Pitch bend sensitivity
	16383	Reset

dat2	Second MIDI data byte as defined by the MIDI specification.  If not required, set to zero.

RETURNS	MIDImessage returns -1 if the channel is out of range, -2 if an unsupported command is specified, or -3 if InitPatchBank has not been called.  Otherwise, 0 is returned if no error.

SEE ALSO	InitPatchBank

EXAMPLE	Turn on MIDI note 60 with a velocity of 64 on channel 2 and set the volume of channel 3 to 100.

MIDImessage (NOTEON | 1, 60, 64);
MIDImessage (CONTROLLER | 2, VOL, 100);
/* MIDI constants NOTEON, CONTROLLER, and VOL are defined in 
   ARIAMIDI.H */

OperatorCancelXE "DOS API Functions:OperatorCancel"§XE "Operator"§

PURPOSE	Use OperatorCancel to immediately shut off an active Aria Synthesizer operator.  The release phases of the operator's wave parameter table will be ignored.

SYNTAX	int OperatorCancel ( unsigned op );

op		The operator number to cancel (0 to 31)

RETURNS	OperatorCancel returns 1 if the operator number is out of range or -1 if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	OperatorOn, OperatorOff

EXAMPLE	Cancel operator number 23.

OperatorCancel (23);

OperatorOffXE "DOS API Functions:OperatorOff"§

PURPOSE	Use OperatorOff to turn off an active Aria Synthesizer operator.  The release phases of the operator's wave parameter table will be activated.

SYNTAX	int OperatorOff ( unsigned op );

op		The operator number to turn off (0 to 31)

RETURNS	OperatorOff returns 1 if the operator number is out of range or -1 if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	OperatorOn, OperatorCancel

EXAMPLE	Turn off operator number 17.

OperatorOff (17);

OperatorOnXE "DOS API Functions:OperatorOn"§

PURPOSE	Use OperatorOn to turn on an Aria Synthesizer operator.  The first stage of the operator's wave parameter table will be activated.

SYNTAX	int OperatorOn (	unsigned op, unsigned wpt, unsigned wave, 	unsigned amp, unsigned pitch, unsigned size,
	unsigned type);

op	The operator number to turn off (0 to 31)

wpt	The wave parameter number (0 to 79)

wave	The wave block number (0x0000 to 0xFFFF)

amp	The maximum operator amplitude (0 to 0x7FFF)

pitch	The frequency value (xxyy in hex):
	xx = Integer portion (00h to FEh)
	yy = Fractional portion (00h to FFh)

The frequency value is computed using the following equations:

xx = (int)

Note:	Freq (in Hz) is the desired output frequency.
Base (in Hz) is the frequency of the Wave Block when output at 22.05 KHz:

	Base = 	where, Size is the number of 256 byte Wave Blocks

size	The number of 256 byte Wave Blocks to output (1 to 256)

type	The waveform type:	0 = looping waveform 
	1 = single play waveform

RETURNS	OperatorOn can return out of range errors 1 through 7 corresponding to the 7 input arguments.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	OperatorOff, OperatorCancel

EXAMPLE	Turn on operator 14 with a pitch of 440.0 Hz, using wave parameter 4, wave block 2 (looping sine wave) and a maximum amplitude of 0x7FFF.

OperatorOn (14, 4, 2, 0x7FFF, 0x0A37, 2, 0)

putMem16XE "DOS API Functions:putMem16"§XE "RAM Locations"§

PURPOSE	Use putMem16 to write a word value to DSP RAM.  This can be used to set control flags and write PCM data to the audio buffers.  A list of defined DSP RAM address locations can be found in Section 3.2.2, Memory Layout.

SYNTAX	void putMem16 ( unsigned addr, unsigned dat );

addr	The DSP RAM address.

dat	The DSP data word to write

SEE ALSO	getMem16

EXAMPLE	Clear the DSP task initialization flag prior to loading a DSP application.

putMem16 (0x6102, 0);

RemoveAriaIntXE "DOS API Functions:RemoveAriaInt"§XE "Interrupts"§

PURPOSE	Use RemoveAriaInt to remove the interrupt service routine that handles digital recording and playback.  This must be called before exiting an application if InstallAriaInt has been used.

SYNTAX	void RemoveAriaInt ( void );

SEE ALSO	InstallAriaInt

EXAMPLE	Install interrupt handler and play a preloaded sound.

unsigned char far *sndbuffer;   /* pointer to sound */
int ch=0;

SystemInit (ARIA_SYNTH);
if (GetInterrupt() > 0)
	{
	InstallAriaInt();
	SetAudioFormat (SR22KHZ | MONO | EIGHTBIT);
	StartPlayback (ch);

/* wait until finished */
	while (GetAudioPosition (ch) != 0L) 
		;
	RemoveAriaInt();
	}

SelectADCXE "DOS API Functions:SelectADC"§XE "ADC"§

PURPOSE	Use SelectADC to switch the Aria analog-to-digital convertor (ADC) on or off.  The default state upon power up is ON.

SYNTAX	int SelectADC ( unsigned mode );

mode	0 = ADC off
1 = ADC on

RETURNS	SelectADC returns 1 if the mode value is out of range or -1 if the DSP is not responding.  Otherwise, 0 is returned if no error.

EXAMPLE	Turn the Aria ADC off.

SelectADC (0);

SelectDSPAppXE "DOS API Functions:SelectDSPApp"§XE "DSP Applications"§

PURPOSE	Use SelectDSPApp to load and unload Aria DSP applications.  Typical DSP applications are Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§, reverbXE "Reverb"§ and QSound Virtual AudioXE "QSound Virtual Audio"§.

SYNTAX	int SelectDSPApp ( unsigned app, int load, unsigned char far *addr );

app	The DSP application ID:
	0	= Aria Synthesizer and PCM audio functions
	1	= Sound Blaster emulation
	2-4	= Reserved
	5	= Aria Listener speech recognition
	6	= Undefined
	7	= Reverb effects
	8	= QSound Virtual Audio effects
	9-15	= Undefined

load	0 = Unload the DSP application
1 = Load the DSP application

addr	Far pointer to DSP application loaded in PC memory.
If this parameter is set to NULL, the function will attempt to load the specified application from Aria system ROM.

RETURNS	SelectDSPApp returns 1 if the application ID argument is invalid, 2 if the application is already loaded when another load call is attempted, 3 or 4 if the DSP application specified by the address pointer is invalid.  

-1 is returned if the DSP is not responding, -2 if Aria is not in the proper mode to load an application, -3 if the system would become overallocated, or -4 if the application is not available.  Otherwise, 0 is returned if no error.

EXAMPLE	Load reverb application from Aria system ROM.

if (SelectDSPApp (REVERB_ID, 1, NULL) == -4)

printf ("Reverb not available in Aria system ROM");

sendDSPXE "DOS API Functions:sendDSP"§XE "COMMANDS"§

PURPOSE	Use sendDSP to send a single command word to the Aria DSP operating system.  Multiple sendDSP calls can be used to send a low-level command sequence to the Aria DSP.

SYNTAX	int sendDSP ( unsigned cmd );

cmd	The command data word to send to the 	Aria DSP.

RETURNS	sendDSP returns 0 if the command was send with no errors.  1 is returned if the DSP is not responding.

EXAMPLE	Send a command string to set the Aria Synthesizer volume to maximum.

unsigned status=0;

_disable ();	/* Disable PC interrupts */
status |= sendDSP (0x0029);	/* Send command ID */
status |= sendDSP (0x7FFF);	/* Send left channel volume */
status |= sendDSP (0x7FFF);	/* Send right channel volume */
status |= sendDSP (0xFFFF);	/* Send terminator */
_enable ();	/* Enable PC interrupts */

if (status)
	printf ("\nERROR - DSP not responding!");

SetAudioFormatXE "DOS API Functions:SetAudioFormat"§

PURPOSE	Use SetAudioFormat to specify the digital audio format for recording and playback.  This must be done prior to starting a record or playback function.

SYNTAX	int SetAudioFormat ( unsigned format );

format	The digital audio format.  This can be a combination of the following defined constants ORed together (found in ARIA.H):

SR11KHZ	11.025 kHz sample rate or
SR22KHZ	22.05 kHz sample rate or
SR44KHZ	44.1 kHz sample rate

	ORed with

MONO	Monophonic sampling or
STEREO	Stereophonic sampling

	ORed with

EIGHTBIT	8-bit  unsigned PCM data format or
SIXTEENBIT	16-bit, signed, LSB first PCM data format or
ADPCM	Sierra ADPCM compression format (16 bit to 4 bit)
MULAW	m-Law compression (13 bit to 8 bit)
ALAW	A-Law compression (13 bit to 8 bit)

Default format is (SR22KHZ | MONO | EIGHTBIT)

RETURNS	SetAudioFormat returns 1 if the format argument is invalid, -1 if the DSP is not responding, or -2 if a digital audio channel is currently active.  Otherwise, 0 is returned if no error.

NOTE	Playback and recording of ADPCM data is only supported at a sample rate of 22 Khz.

EXAMPLE	Set the system for playing 16-bit, stereo digital audio at 44.1 kHz sampling rate.

/* Synthesis is not available at 44.1 kHz, so set system playback mode */

SetPlaybackMode (NOSYNTH);
SetAudioFormat (SR44KHZ | STEREO | SIXTEENBIT);

SetAudioTransferXE "DOS API Functions:SetAudioTransfer"§

PURPOSE	Use SetAudioTransfer to signal the completion of a digital audio packet transfer.  This command is not necessary if using the library's interrupt service routine (see InstallAriaInt).

SYNTAX	int SetAudioTransfer ( void );

RETURNS	SetAudioTransfer returns -1 if the DSP is not responding or 0 if no error.

SetAuxVolumeXE "DOS API Functions:SetAuxVolume"§XE "CD-ROM Volume"§XE "Volume:CD-ROM"§XE "Auxiliary Volume"§XE "Volume:Auxiliary"§

PURPOSE	Use SetAuxVolume to adjust the auxiliary CD-ROM audio volume.  Both the CD-ROM monitor volume and the CD-ROM recording volume will be effected.

SYNTAX	int SetAuxVolume ( unsigned left, unsigned right );

left		Left channel aux volume (0 to 0x7FFF)

right	Right channel aux volume (0 to 0x7FFF)

RETURNS	SetAuxVolume returns 1 if the left volume argument is out of range or 2 if the right volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetMixerConfig

EXAMPLE	Set the CD-ROM stereo audio volume to 50%.

SetAuxVolume (0x3FFF, 0x3FFF);

SetChannelPanXE "DOS API Functions:SetChannelPan"§

PURPOSE	Use SetChannelPan to adjust a digital audio playback channel's stereo pan.

SYNTAX	int SetChannelPan ( unsigned ch, unsigned pan );

ch	The playback channel to adjust (0 to 3)

pan	The stereo pan setting (0 left to 64 center to 127 right)

RETURNS	SetChannelPan returns 1 if the channel argument is out of range or 2 if the pan value is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetChannelVolume

EXAMPLE	Set playback channel 0 to full left pan and playback channel 1 to full right pan.

SetChannelPan (0, 0);
SetChannelPan (1, 127);

SetExtVolumeXE "DOS API Functions:SetExtVolume"§XE "Microphone Volume"§XE "Volume:Microphone"§XE "External Volume"§XE "Volume:External"§

PURPOSE	Use SetExtVolume to adjust the microphone input volume.  Both the microphone monitor volume and the microphone recording volume will be effected.

SYNTAX	int SetExtVolume ( unsigned left, unsigned right );

left		Left channel ext volume (0 to 0x7FFF)

right	Right channel ext volume (0 to 0x7FFF)

RETURNS	SetExtVolume returns 1 if the left volume argument is out of range or 2 if the right volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetMasterVolume, SetMixerConfig

EXAMPLE	Set the external stereo audio volume to 50%.

SetExtVolume (0x3FFF, 0x3FFF);

SetChannelVolumeXE "DOS API Functions:SetChannelVolume"§XE "Volume:Playback Channel"§

PURPOSE	Use SetChannelVolume to adjust a digital audio playback channel's volume.

SYNTAX	int SetChannelVolume ( unsigned ch, unsigned vol );

ch		The playback channel to adjust (0 to 3)

vol		The volume setting (0 to 0x7FFF)

RETURNS	SetChannelVolume returns 1 if the channel argument is out of range or 2 if the volume value is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetChannelPan

EXAMPLE	Set playback channel 0 to full volume and playback channel 1 to half volume.

SetChannelVolume (0, 0x7FFF);
SetChannelVolume (1, 0x3FFF);

SetLFOdepthXE "DOS API Functions:SetLFOdepth"§

PURPOSE	Use SetLFOdepth to adjust the amount of LFO effect for an active operator.

SYNTAX	int SetLFOdepth ( unsigned wpt, unsigned type, unsigned depth );

wpt	The wave parameter table number (0 to 79)

type	The LFO type:
	0	Pitch LFO (vibrato)
	1	Amplitude LFO (tremolo)
	2	Pan LFO

depth	The new LFO depth (0 to 255)

RETURNS	SetLFOdepth returns 1 if the wave parameter table number argument is out of range, 2 if the LFO type argument is out of range or 3 if the depth argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetLFOrate

EXAMPLE	Set the pitch LFO depth of wave parameter 5 to maximum.

SetLFOdepth (5, 0, 255);

SetLFOrateXE "DOS API Functions:SetLFOrate"§

PURPOSE	Use SetLFOrate to adjust the rate or speed of an LFO generator.

SYNTAX	int SetLFOrate ( unsigned LFOnum, unsigned rate );

LFOnum	The LFO generator number (0 to 15)

rate	The LFO frequency value (xxyy in hex):
		xx = LFO rate (int. portion)  00h to 3Fh
		yy = LFO rate (frac. portion)  00h to FFh

LFO rate is computed using the following equations:

xx = (int)

Note:  Frequency is in hertz.

RETURNS	SetLFOrate returns 1 if the LFO number is out of range or 2 if the rate argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetLFOdepth

EXAMPLE	Set LFO generator number 12 to a rate of 52.0 Hz.

float freq = 52.0;
unsigned LFOint, LFOfrac;

LFOint = (unsigned) (freq / 172.3);
LFOfrac = (unsigned) (((freq / 172.3) - LFOint) * 512.0);
SetLFOrate (12, (LFOint << 8) | LFOfrac);
XE "Low Frequency Oscillator" \r "LFO4"§

SetListenerModeXE "DOS API Functions:SetListenerMode"§XE "Aria Listener"§XE "Speech Recognition"§

PURPOSE	Use SetListenerMode to inform the Aria API that the Aria Listener speech recognition application is active or inactive.  If the SelectDSPApp function has been used to load Aria Listener, this function is not required.

SYNTAX	int SetListenerMode ( unsigned mode );

mode	0 = Aria Listener off
1 = Aria Listener on

RETURNS	SetListenerMode returns 1 if the mode argument is invalid, -1 if the DSP is not responding, or -2 if Aria Listener has not been loaded.  Otherwise, 0 is returned if no error.

EXAMPLE	Inform the system that Aria Listener speech recognition has been switched on.

SetListenerMode (1);

SetMasterModeXE "DOS API Functions:SetMasterMode"§XE "Monitor"§	SC18075 Mixer Only

PURPOSE	Use SetMasterMode to set the output monitor mode to mono or stereo and to select the mixed output recording signal.  The mixed output recording signal can be used to record the final output via the Auxiliary input channel.

SYNTAX	int SetMasterMode (unsigned mode);

mode	Low-order byte:
0 = stereo monitoring (default)
1 = mono monitoring	

High-order byte:
0 = disconnect mixed output for recording (default)
1 = connect mixed output for recording

RETURNS	SetMasterMode returns 1 if the mode argument is invalid, -1 if the DSP is not responding, or -2 if the command is not available (available with SC18075 mixer only).  Otherwise, 0 is returned if no error.

EXAMPLE	Set the master monitor mode to monophonic and disconnect the mixed output recording signal.

SetMasterMode (1);

SetMasterVolumeXE "DOS API Functions:SetMasterVolume"§XE "Master Volume"§XE "Volume:Master"§

PURPOSE	Use SetMasterVolume to adjust the overall output of the audio device.  Left and right channel outputs may be adjusted independently.

When this function is used for systems without the SC18075 mixer chip, only the output of the Aria Synthesizer and PCM audio channels will be affected.

SYNTAX	int SetMasterVolume ( unsigned left, unsigned right );

left		The left channel volume (0 to 0x7FFF)

right	The right channel volume (0 to 0x7FFF)

RETURNS	SetMasterVolume returns 1 if the left volume argument is out of range or 2 if the right volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetPlaybackVolume, SetSynthVolume

EXAMPLE	Set the master volume of both channels to 25% of maximum.

SetMasterVolume (0x1FFF, 0x1FFF);

SetMixerConfigXE "DOS API Functions:SetMixerConfig"§XE "Mixer"§XE "Monitor"§

PURPOSE	Use SetMixerConfig to configure the recording source, mixer input levels and the recording monitor switches.  Microphone input may be switched between microphone and line in level.  Inputs may be mixed with the output if the monitor enables are selected.

SYNTAX	int SetMixerConfig (	unsigned source, 
	unsigned mic, unsigned micmon,
	unsigned aux, unsigned auxmon );

source	The recording source (must be one of the following):

0x0000	ADC off
0x0001	Microphone*
0x0002	CD-ROM audio (AUX2)*
0x0004	Aria Synthesizer/PCM audio output*
0x0008	External
0x0010	Auxiliary 1 (mixed output)
0x0020	PC Speaker/Telephone input
0x0005	Microphone and Synth/wave output
0x0003	Microphone and CD-ROM audio
0x0021	Microphone and PC Speaker/Telephone input
0x0014	Auxiliary 1 and Synth/wave output
0x0012	Auxiliary 1 and CD-ROM audio
0x0030	Auxiliary 1 and PC Speaker/Telephone input
0x000C	External and Synth/wave output
0x000A	External and CD-ROM audio
0x0028	External and PC Speaker/Telephone input

*These source parameters are the only options available for Aria systems without the SC18075 mixer.

mic	The microphone input level selector 
	1 = microphone
	0 = line level
Aria systems with the SC18075 mixer have seperate input sources for microphone vs. external line level.  This parameter is ignored for those systems.

micmon	The external input monitor enable:

0x00 = disable monitor, stereo mode
0x01 = enable monitor, stereo mode
0x10 = disable monitor, mono mode
0x11 = enable monitor, mono mode

aux	The auxiliary 2 (CD-ROM) input level selector
(value is ignored, currently fixed at line level)

auxmon	The auxiliary 2 (CD-ROM) input monitor enable:

0x00 = disable monitor, stereo mode
0x01 = enable monitor, stereo mode
0x10 = disable monitor, mono mode
0x11 = enable monitor, mono mode

RETURNS	SetMixerConfig can return out of range errors 1 through 5 corresponding to the five input arguments.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetMonitorMode

Note	The default settings are microphone level recording with the mic monitor disabled and auxiliary stereo input with the monitor enabled.

EXAMPLE	Set the Aria mixer configuration to the default position described above.

SetMixerConfig (1, 1, 0, 0, 0);

SetMixerVolumeXE "DOS API Functions:SetMixerVolume"§XE "Mixer"§XE "Volume"§	SC18075 Mixer Only

PURPOSE	Use SetMixerVolume to set individual mixer channel volume controls.

SYNTAX	int SetMixerVolume (	unsigned source, unsigned left, 
	unsigned right );

source	Any of the following sources (ORed together):
	0x0001	= Microphone
	0x0002	= CD-ROM audio (AUX2)
	0x0004	= Master volume
	0x0008	= External
	0x0010	= Auxiliary 1 (mixed output)
	0x0020	= PC Speaker/Telephone input

left	Left channel volume (0 to 0x7FFF)

right	Right channel volume (0 to 0x7FFF)

RETURNS	SetMixerVolume returns 1 if the source argument is out of range, 2 if the left volume argument is out of range, or 3 if the right volume argument is out of range.  -1 is returned if the DSP is not responding or -2 if the command is not available (available with SC18075 mixer only).  Otherwise, 0 is returned if no error.

SEE ALSO	SetAuxVolume, SetExtVolume, SetMasterVolume

EXAMPLE	Set the CD-ROM audio volume to 50% of maximum.

SetMixerVolume (2, 0x3FFF, 0x3FFF);

SetMonitorModeXE "DOS API Functions:SetMonitorMode"§XE "Monitor"§	SC18075 Mixer Only

PURPOSE	Use SetMonitorMode to set the monitor mode of the input channels to mono or stereo.

SYNTAX	int SetMonitorMode ( unsigned source, unsigned mode );

source	One of the following input sources:
	0x0001	= Microphone
	0x0002	= CD-ROM audio (AUX2)
	0x0008	= External input
	0x0010	= Auxiliary 1 (mixed output)
	0x0020	= PC Speaker/Telephone input

mode	0 = Stereo monitoring
1 = Mono, left channel
2 = Mono, right channel
3 = Monitor off

RETURNS	SetMonitorMode returns 1 if the source argument is out of range or 2 if the mode argument is out of range.  -1 is returned if the DSP is not responding or -2 if the command is not available (available with SC18075 mixer only).  Otherwise, 0 is returned if no error.

SEE ALSO	SetMixerConfig

EXAMPLE	Set the microphone input to stereo.

SetMonitorMode (1, 0);

SetOperatorCountXE "DOS API Functions:SetOperatorCount"§XE "Operator"§

PURPOSE	Use SetOperatorCount to change the number of available Aria Synthesizer operators.  This can be used to decrease the operator count when running DSP applications that require a large amount of DSP processing power.

SYNTAX	int SetOperatorCount ( unsigned opcount );

opcount	Number of synthesis operators (1 to 32)

RETURNS	SetOperatorCount returns 1 if the operator count is out of range, -1 if the DSP is not responding, or -2 if the specified count is not currently available.  Otherwise, 0 is returned if no error.

SEE ALSO	InitPatchBank, SetPlaybackMode

EXAMPLE	Set the Aria Synthesizer operator count to 20.

SetOperatorCount (20);

SetOperatorPanXE "DOS API Functions:SetOperatorPan"§

PURPOSE	Use SetOperatorPan to adjust the stereo pan position of any active Aria Synthesizer operator.

SYNTAX	int SetOperatorPan ( unsigned op, unsigned pan );

op	The operator number (0 to 31)

pan	The stereo pan value (0 left to 64 center to 127 right)

RETURNS	SetOperatorPan returns 1 if the operator number argument is out of range or 2 if the pan argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

EXAMPLE	Set operator 10 to full left pan, operator 11 to center, and operator 12 to full right pan.

SetOperatorPan (10, 0);
SetOperatorPan (11, 64);
SetOperatorPan (12, 127);

SetOperatorPitchXE "DOS API Functions:SetOperatorPitch"§

PURPOSE	Use SetOperatorPitch to change the pitch (or frequency) of the sound output by an active Aria Synthesizer operator.

SYNTAX	int SetOperatorPitch ( unsigned op, unsigned pitch );

op	The operator number (0 to 31)

pitch	The frequency value (xxyy in hex):
	xx = Integer portion (00h to FEh)
	yy = Fractional portion (00h to FFh)

The frequency value is computed using the following equations:

xx = (int)

Note:	Freq (in Hz) is the desired output frequency.
Base (in Hz) is the frequency of the Wave Block when output at 22.05 KHz.

RETURNS	SetOperatorPitch returns 1 if the operator number argument is out of range or 2 if the pitch argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

EXAMPLE	Set the pitch of operator 4 (Wave Block base frequency 43.066 Hz) to a frequency of 440.0 Hz.

SetOperatorPitch (4, 0x0A37);

SetOperatorVolumeXE "DOS API Functions:SetOperatorVolume"§XE "Volume:Synthesis Operator"§XE "Operator Volume"§

PURPOSE	Use SetOperatorVolume to change the volume (or amplitude) of the sound output by an active Aria Synthesizer operator.

SYNTAX	int SetOperatorVolume ( unsigned op, unsigned vol );

op	The operator number (0 to 31)

vol	The operator volume or amplitude (0 to 0x7FFF)

RETURNS	SetOperatorVolume returns 1 if the operator number argument is out of range or 2 if the volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetSynthVolume

EXAMPLE	Set the volume of operator 7 to maximum and the volume of operator 8 to 50% of maximum.

SetOperatorVolume (7, 0x7FFF);
SetOperatorVolume (8, 0x3FFF);

SetPacketSizeXE "DOS API Functions:SetPacketSize"§

PURPOSE	Use SetPacketSize to change the number of words that comprise a digital audio packet to be sent or received by the Aria system.

SYNTAX	int SetPacketSize ( unsigned size );

size	The number of words in one data packet
(must be 4, 8, 16, 32, 64, 128, 256, or 512)

Default packet size is 512 words.

RETURNS	SetPacketSize returns 1 if the size argument is out of range or -1 if the DSP is not responding.  Otherwise, 0 is returned if no error.

EXAMPLE	Set the transfer packet size to 64.

SetPacketSize (64);

SetPlaybackModeXE "DOS API Functions:SetPlaybackMode"§

PURPOSE	Use SetPlaybackMode to select the synthesis operator mode.  This function sets the number of available Aria Synthesizer operators versus the number of available digital audio channels.  No digital audio channels can be playing or recording when this function is called.

SYNTAX	int SetPlaybackMode ( unsigned mode );

mode	The playback mode to set:

0 = 4 digital audio channels, no synthesis
1 = 2 digital audio channels, 20 synthesis operators
2 = 2 digital audio channels, 32 synthesis operators

The default mode is 2.

RETURNS	SetPlaybackMode returns 1 if the mode argument is invalid, -1 if the DSP is not responding, or -2 if the specified mode is not currently available.  Otherwise, 0 is returned if no error.

EXAMPLE	Set the system for digital audio recording and playback only.

SetPlaybackMode (0);

SetPlaybackVolumeXE "DOS API Functions:SetPlaybackVolume"§XE "Volume:Digital Audio Playback"§

PURPOSE	Use SetPlaybackVolume to adjust the overall volume level of all digital audio playback channels.

SYNTAX	int SetPlaybackVolume ( unsigned left, unsigned right );

left	The left channel volume (0 to 0x7FFF)

right	The right channel volume (0 to 0x7FFF)

RETURNS	SetPlaybackVolume returns 1 if the left volume argument is out of range or 2 if the right volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetMasterVolume, SetSynthVolume

EXAMPLE	Set the audio playback to maximum volume.

SetPlaybackVolume (0x7FFF, 0x7FFF);

SetQSoundModeXE "DOS API Functions:SetQSoundMode"§XE "QSound Virtual Audio"§

PURPOSE	Use SetQSoundMode to select the different QSound Virtual AudioXE "QSound Virtual Audio"§ modes.  The QSound DSP application must be loaded prior to using this command.

SYNTAX	int SetQSoundMode ( unsigned mode );

mode	The QSound mode:
		0 = Off
		1 = Apply effect to Aria PCM audio only
		2 = Apply effect to Aria Synthesizer only
		3 = Apply effect to both Aria Synthesizer and PCM audio

RETURNS	SetQSoundMode returns 1 if the mode argument is out of range, -1 if the DSP is not responding, or -2 if the QSound DSP application is not loaded.  Otherwise, 0 is returned if no error.

SEE ALSO	SelectDSPApp

EXAMPLE	Apply QSound effects to Aria Synthesizer only, assuming that the DSP application is already loaded.

SetQSoundMode (2);

SetRecordingVolumeXE "DOS API Functions:SetRecordingVolume"§XE "Volume:Recording"§XE "Recording Volume"§

PURPOSE	Use SetRecordingVolume to adjust the digital audio recording level(s).  Separate left and right channel levels can be specified, corresponding to stereo recording channels.  For monophonic recording, only the left channel volume is used.

SYNTAX	int SetRecordingVolume (	unsigned source, unsigned left, 
	unsigned right );

source	The input source to adjust.  Use one of the following defined constants (from ARIA.H):
	EXTREC	= Microphone input
	AUXREC	= Auxiliary input

left	The left channel volume (0 to 0x7FFF)

right	The right channel volume (0 to 0x7FFF)

RETURNS	SetRecordingVolume returns 1 if the source value is out of range, 2 if the left volume argument is out of range or 3 if the right volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

EXAMPLE	Set the stereo microphone recording levels to maximum volume.

SetRecordingVolume (EXTREC, 0x7FFF, 0x7FFF);

SetReverbChannelXE "DOS API Functions:SetReverbChannel"§

PURPOSE	Use SetReverbChannel to set the level of reverberation effect applied to an individual Wave Parameter Table (WPT) in the Aria Synthesizer.  The reverb DSP application must be loaded prior to using this command.

SYNTAX	int SetReverbChannel ( unsigned wpt, unsigned level );

wpt	The Wave Parameter Table number (0 to 0x4F)

level	The WPT reverb level (0 to 0x7FFF)

RETURNS	SetReverbChannel returns 1 if the wpt argument is out of range, 2 if the level argument is out of range, -1 if the DSP is not responding, or -2 if the reverb DSP application is not loaded.  Otherwise, 0 is returned if no error.

SEE ALSO	SelectDSPApp, SetReverbLevel, SetReverbMode

EXAMPLE	Turn off the reverb effect for WPTs 0 to 4.

unsigned i;

for (i = 0; i < 5; ++i)
	SetReverbChannel (i, 0);

NOTES	It is recommended that the function MIDImessage be used to control individual MIDI channel reverb levels instead of SetReverbChannel.

SetReverbLevelXE "DOS API Functions:SetReverbLevel"§

PURPOSE	Use SetReverbLevel to set the amount or level of reverberation effect.  The reverb DSP application must be loaded prior to using this command.

SYNTAX	int SetReverbLevel ( unsigned left, unsigned right );

left	The left channel reverb level (0 to 0x7FFF)

right	The right channel reverb level (0 to 0x7FFF)

RETURNS	SetReverbLevel returns 1 if the left channel argument is out of range, 2 if the right channel argument is out of range, -1 if the DSP is not responding, or -2 if the reverb DSP application is not loaded.  Otherwise, 0 is returned if no error.

SEE ALSO	SelectDSPApp, SetReverbMode

EXAMPLE	Set the reverb level to 25% on the left channel and 75% on the right channel.  Assume that the reverb DSP application has been loaded and that a reverb mode has been selected.

SetReverbLevel (0x1FFF, 0x5FFF);

SetReverbModeXE "DOS API Functions:SetReverbMode"§

PURPOSE	Use SetReverbMode to select the different reverberation effect modes.  The reverb DSP application must be loaded prior to using this command.

SYNTAX	int SetReverbMode ( unsigned mode );

mode	The reverb mode:
		1 = Room 1
		2 = Room 2
		3 = Room 3
		4 = Hall 1
		5 = Hall 2
		6 = Plate echo
		7 = Delay

RETURNS	SetReverbMode returns 1 if the mode argument is out of range, -1 if the DSP is not responding, or -2 if the reverb DSP application is not loaded.  Otherwise, 0 is returned if no error.

SEE ALSO	SelectDSPApp, SetReverbLevel

EXAMPLE	Apply Hall 1 reverb effect to Aria output, assuming that the DSP application is already loaded.

SetReverbMode (4);

SetReverbWaveXE "DOS API Functions:SetReverbWave"§

PURPOSE	Use SetReverbWave to set the level of reverberation effect applied to all digital audio output channels.  The reverb DSP application must be loaded prior to using this command.

SYNTAX	int SetReverbWave ( unsigned level );

level	The digital audio reverb level (0 to 0x7FFF)

RETURNS	SetReverbWave returns 1 if the level argument is out of range, -1 if the DSP is not responding, or -2 if the reverb DSP application is not loaded.  Otherwise, 0 is returned if no error.

SEE ALSO	SelectDSPApp, SetReverbLevel, SetReverbMode

EXAMPLE	Set the digital audio reverb effect level to 50%.

SetReverbWave (0x3FFF);XE "Reverb" \r "Reverb"§

SetSynthVolumeXE "DOS API Functions:SetSynthVolume"§XE "Volume:Synthesis"§

PURPOSE	Use SetSynthVolume to adjust the overall volume level for all Aria Synthesizer operators.

SYNTAX	int SetSynthVolume ( unsigned left, unsigned right );

left	The left channel volume (0 to 0x7FFF)

right	The right channel volume (0 to 0x7FFF)

RETURNS	SetSynthVolume returns 1 if the left volume argument is out of range or 2 if the right volume argument is out of range.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetMasterVolume, SetPlaybackVolume

EXAMPLE	Set the stereo synthesis volume to 50% of maximum.

SetSynthVolume (0x3FFF, 0x3FFF);

SetToneControlXE "DOS API Functions:SetToneControl"§XE "Tone Control"§	SC18075 Mixer Only

PURPOSE	Use SetToneControl to set the treble and bass tone controls for the Aria system.

SYNTAX	int SetToneControl ( unsigned left, unsigned right );

left	Left channel tone control.  Treble value must be placed in the high-order byte, bass value must be placed in the low-order byte:

0	= Flat response
1 to 7	= Maximum cut to minimum cut frequency
8	= Flat response
9 to 15	= Minimum boost to maximum boost frequency

right	Right channel tone control.  Parameter is same as described above.

RETURNS	SetToneControl returns 1 if the left control argument is out of range or 2 if the right control argument is out of range.  -1 is returned if the DSP is not responding or -2 if the command is not available (available with SC18075 mixer only).  Otherwise, 0 is returned if no error.

EXAMPLE	Set the bass tone controls to maximum and the treble to flat response.

SetToneControl (0x080F, 0x080F);

SetWaveParameterXE "DOS API Functions:SetWaveParameter"§XE "Wave Parameter Table"§

PURPOSE	Use SetWaveParameter to initialize a wave parameter table for use with an Aria Synthesizer operator.  The wave parameter table defines the 8 stage amplitude envelope, the LFO depth values, and the default operator pan setting.

SYNTAX	int SetWaveParameter (	unsigned wpt, struct envstruct far *env, 
	struct wptLFOstruct far *lfo, unsigned pan );

wpt	The wave parameter table number (0 to 79)

env	A pointer to an array of 8 amplitude envelope structures (defined in ARIA.H):

struct envstruct
	{
	unsigned index : 6;	/* 0 to 63 */
	unsigned length : 10;	/* 0 to 1022 */
	} env[8];

lfo	A pointer to an array of 3 LFO structures (defined in ARIA.H):

struct wptLFOstruct
	{
	unsigned LFOnum;	/* 0 to 15 */
	unsigned LFOdepth;	/* 0 to 255 */
	} lfo[3];

lfo[0] is the Vibrato LFO definition
lfo[1] is the Tremolo LFO definition
lfo[2] is the Pan LFO definition

pan	The default pan setting (0 to 127)

RETURNS	SetWaveParameter can return out of range errors 1 through 16 corresponding to the input arguments (each envelope array element and each LFO structure item is counted).  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	TableModify

EXAMPLE	Set up wave parameter table number 0 with sample operator definition information.  Set LFO number 1 to be a tremolo generator with a depth of 50.

struct envstruct env[8];
struct wptLFOstruct lfo[3];

env[0].index = 24;	/* stage 0 index */
env[0].length = 8;	/* stage 0 length */
	"		"
env[7].index = 59;	/* stage 7 index */
env[7].length = 100;	/* stage 7 length */
lfo[0].LFOnum = 0;	/* vibrato LFO number */
lfo[0].LFOdepth = 0;	/* vibrato LFO depth */
lfo[1].LFOnum = 1;	/* tremolo LFO number */
lfo[1].LFOdepth = 50;	/* tremolo LFO depth */
lfo[2].LFOnum = 0;	/* pan LFO number */
lfo[2].LFOdepth = 0;	/* pan LFO depth */

SetWaveParameter (0, (struct envstruct far *) &env[0], 
		(struct wptLFOstruct far *) &lfo[0], 64);

StartPlaybackXE "DOS API Functions:StartPlayback"§XE "Playback"§

PURPOSE	Use StartPlayback to start digital audio playing on a selected channel.  InstallAriaInt must be called prior to using this function.  If the sample size is greater than the size of the buffer used, the function will continue playing audio by cycling back to the beginning of the buffer.  In this case, the calling application is required to refresh the buffer data at regular intervals.

SYNTAX	int StartPlayback (	unsigned ch, unsigned char far *ptr,
	unsigned long bufsize, 
	unsigned long sampsize );

ch	The audio playback channel (0 to 3)

ptr	A far pointer to a buffer containing the digital audio data to play.

bufsize	The size (in bytes) of the buffer referred to by ptr.  Must be a multiple of (packet size * 2)

sampsize	The size (in bytes) of the entire sample

RETURNS	StartPlayback returns 1 if the channel argument is out of range, 2 if the buffer pointer is NULL, 3 if the buffer size is zero or not a multiple of (packet size * 2), or 4 if the sample size is zero.  -1 is returned if the DSP is not responding and -2 is returned if the selected channel is already active.  Otherwise, 0 is returned if no error.

SEE ALSO	InstallAriaInt, RemoveAriaInt, StopPlayback, GetBufferPosition, GetAudioPosition

EXAMPLE	Install interrupt handler and play a preloaded sound that is 16 Kbytes in length.

unsigned char far *sndbuffer;   /* pointer to sound */
int ch=0;

SystemInit (ARIA_SYNTH);
if (GetInterrupt() > 0)
	{
	InstallAriaInt();
	SetAudioFormat (SR22KHZ | MONO | EIGHTBIT);
	StartPlayback (ch, sndbuffer, 16384L, 16384L);

/* wait until finished */
	while (GetAudioPosition (ch) != 0L) 
		;
	RemoveAriaInt();
	}

StartRecordingXE "DOS API Functions:StartRecording"§XE "Recording"§

PURPOSE	Use StartRecording to begin recording digital audio from an external source.  InstallAriaInt must be called prior to using this function.  If the sample size is greater than the size of the buffer used, the function will continue recording audio by cycling back to the beginning of the buffer.  In this case, the calling application is required to write the buffer data to disk at regular intervals.

SYNTAX	int StartRecording (	unsigned ch, unsigned char far *ptr, 
	unsigned long bufsize, 
	unsigned long sampsize );

ch	The audio recording channel (0 or 1)

ptr	A far pointer to a buffer to receive the digital audio data.

bufsize	The number of bytes in the ptr buffer
Must be a multiple of (packet size * 2)

sampsize	The number of bytes of audio data to record

RETURNS	StartRecording returns 1 if the channel argument is out of range, 2 if the buffer pointer is NULL, 3 if the buffer size is zero or not a multiple of (packet size * 2), or 4 if the sample size is zero.  -1 is returned if the DSP is not responding and -2 is returned if the selected channel is already active.  Otherwise, 0 is returned if no error.

SEE ALSO	InstallAriaInt, RemoveAriaInt, StopRecording

EXAMPLE	Install interrupt handler and record a sound from the external microphone into a buffer that is 16 Kbytes in length.

unsigned char far *sndbuffer;   /* pointer to sound */
int ch=0;

SystemInit (ARIA_SYNTH);
if (GetInterrupt() > 0)
	{
	InstallAriaInt();
	SetAudioFormat (SR22KHZ | MONO | EIGHTBIT);
	SetMixerConfig (EXTREC, 1, 0, 0, 0);
	StartRecording (ch, sndbuffer, 16384L, 16384L);

/* wait until finished */
	while (GetAudioPosition (ch) != 0L) 
		;
	RemoveAriaInt();
	}

StopPlaybackXE "DOS API Functions:StopPlayback"§XE "Playback"§

PURPOSE	Use StopPlayback to halt the playback of an active digital audio channel.

SYNTAX	int StopPlayback ( unsigned ch );

ch		The playback channel to stop (0 to 3)

RETURNS	StopPlayback returns 1 if the channel argument is out of range, -1 if the DSP is not responding, or -2 if the specified channel is not active.  Otherwise, 0 is returned if no error.

SEE ALSO	StartPlayback

EXAMPLE	Turn off playback channel 0.

StopPlayback (0);

StopRecordingXE "DOS API Functions:StopRecording"§XE "Recording"§

PURPOSE	Use StopRecording to halt the recording on an active digital audio channel.

SYNTAX	int StopRecording ( unsigned ch );

ch		The recording channel to stop (0 or 1)

RETURNS	StopRecording returns 1 if the channel argument is out of range, -1 if the DSP is not responding, or -2 if the specified channel is not active.  Otherwise, 0 is returned if no error.

SEE ALSO	StartRecording

EXAMPLE	Turn off recording channel 0.

StopRecording (0);

SystemInitXE "DOS API Functions:SystemInit"§XE "System Initialization"§

PURPOSE	Use SystemInit to initialize the Aria system and specify which DSP application to run.  SystemInit also determines the base port address for Aria functions and the DMA channel.  If successful, the base port address is placed in global variable dspbase and used by subsequent library function calls.  The DMA channel number is stored in global variable dmachan.  Global variable version will also be initialized if SystemInit is successful.

SYNTAX	int SystemInit ( unsigned app );

app	The DSP application to execute.  It can be one of the following defined constants (in ARIA.H):

DEFAULT_SYNTH	Sound Blaster emulation (default)

ARIA_SYNTH	Aria PCM Audio and Aria Synthesizer mode

RETURNS	SystemInit returns 1 if the app argument is out of range, -1 if the DSP is not responding, -2 if the system could not initialize, -3 if the Aria system is not found, or -4 if the DMA channel is not detected.  Otherwise, 0 is returned if no error.

EXAMPLE	Return the system to the default state (Sound Blaster emulation mode).

SystemInit (DEFAULT_SYNTH);

TableModifyXE "DOS API Functions:TableModify"§

PURPOSE	Use TableModify to selectively change an Aria system data table.  This includes wave parameter tables and the envelope generation multiplier table.

SYNTAX	int TableModify (	unsigned type, unsigned table, unsigned offset, 
	unsigned value );

type	The type of data table to modify.  It can be one of the following:

	0	Wave parameter table
	1	Envelope generation multiplier table

table	The table number to modify:

	0 to 79	for wave parameter tables
	0	for envelope generation table

offset	The word offset (address) in the table to modify:

For wave parameter tables
	0 to 7	Envelope stage parameters
	14	Right pan amplitude
	15	Left pan amplitude

For envelope generation multiplier table
	0 to 63	Multiplier values for envelope scaling 
		(corresponds to envelope index values)

value	The new data value to set (NOTE:  0xFFFF is not allowed)

RETURNS	TableModify can return out of range errors 1 through 4 corresponding to the four input arguments.  -1 is returned if the DSP is not responding.  Otherwise, 0 is returned if no error.

SEE ALSO	SetWaveParameter

EXAMPLE	Change the first envelope stage of wave parameter table number 70 to 0x018A.

TableModify (0, 70, 0, 0x018A);XE "DOS API Functions" \r "APIFunctions"§

XE "DOS Application Interface" \r "DOSAPI"§

## 1.30	ARIA DSP APPLICATION MANAGER§

This section describes the Aria DSP Application Manager (DSPAM) interface.  DSPAM is a system device driver that may be loaded by the PC's CONFIG.SYS file during power-up.  It provides a low-level Application Programming Interface (API) that can handle all DSP management functions.  These functions include providing DSP status information, loading and unloading DSP applications, and ensuring that the DSP does not become overallocated.

This section covers all details regarding programming with the DSPAM API, including function entry parameters, processing description, and function exit codes.

Please note:  This interface may only be used by programs and drivers that do not link to the DOS API Libraries outlined in Sections 6.1 and 6.2.

The following API functions are provided:

| No. | Name | Description |
| --- | --- | --- |
| 00h | Install Check | Determine if and where the driver is installed |
| 01h | Get Entry Point | Get ISR entry point address for direct calls |
| 02h | Uninstall | Uninstall the driver |
| 03h | Request Pop-up | Request pop-up screen display |
| 04h | Determine Chain | Determine other chained interrupt routines |
| 10h | Get Handle Flag | Get DSP handle flag indicating currently loaded DSP applications |
| 11h | Query Application ID | Get DSP application ID name |
| 12h | Query Application Info | Get information on a specific DSP application |
| 13h | Load DSP Application | Load a new DSP application |
| 14h | Unload DSP Application | Unload a DSP application |
| 15h | Get DSP Utilization | Return DSP system utilization |
| 16h | Get System Mode | Get DSP system mode |
| 17h | Set System Mode | Set DSP system mode |
| 18h | Set Callback | Register or remove callback address |

### 1.3.1	Command Line Options

Several optional command line parameters may be specified upon driver installation.  The following is an example with defaults shown:

DEVICE=C:\DSPAM.SYS /A290 /I10 /D5 /T1 /PC:\DSPAM.CFG

where,	/A290 indicates the Aria base port address (280h, 290h, 2A0h, or 2B0h)
/I10 indicates the Aria interrupt number (10, 11, or 12)
/D5 indicates the Aria DMA channel (5 or 6)
/T1 indicates the Aria system version (1 or 2)
/Pfilename indicates a path and filename for the configuration file (see section 6.3.2, "DSP Configuration File" for more information)

### 1.3.2	DSP Configuration File

The DSPAM.SYS driver requires that a DSP Configuration File, "DSPAM.CFG," to be present in the DOS directory search path.  The format of this text file is as follows:

ID_NAME1=FILENAME1
ID_NAME2=FILENAME2

"      "
     "      "

where, ID_NAMEn can be any identifier name (no spaces, up to 8 characters).  FILENAMEn must be a path and filename that specifies an Aria downloadable DSP application.  An example configuration file may look like this:

QSOUND=C:\ARIA\QSVA25.DSP

REVERB=C:\ARIA\REVERB.DSP

### 1.3.3	API Calling Procedure and Functions

Software Interrupt Communication

The Aria DSPAM.SYS driver installs an Interrupt Service Routine at the Alternate Multiplex Interrupt (2Dh).  Communication with the DSPAM API is usually done using the DOS 2Dh software interrupt.  However, some DSPAM functions, which perform disk I/O, may require direct calls from an application.

An application must first determine if and where the DSPAM API is loaded.  To to this, the API function "Install Check" must be called.  Install Check returns a unique signature string for each API that may be using the 2Dh interrupt.  Several calls may be necessary to locate the DSPAM API in the interrupt chain.

Once DSPAM is found, an application communicates by loading PC registers with input data and making an INT 2Dh interrupt call or a direct call using the address returned by the "Get Entry Point" function.  Return information is passed back to the application via registers.  Some functions such as "Load DSP Application" may require that the entry point address be called directly.

Application Callbacks

An application may register a callback address with the DSPAM device driver.  Callbacks are useful for determining when the Aria system has changed playback modes or loaded new DSP applications.  The callback address must point to an interrupt service routine (ISR).  This ISR is not allowed to callback a DSPAM function.

Please note that the entire DSPAM device driver is non-reentrant.  In other words, it may not be called from within any external ISR.

Install Check	Function 00h

This function performs an installation check to determine if the specified ISR is in the interrupt chain.  Up to 256 different ISRs are possible, so this function must be called until the DSPAM ISR is found.  Each ISR is installed at a different multiplex number and has a unique signature string.

An application should scan all multiplex numbers starting with 00h.  For multiplex numbers which are in use, the program should compare the first 16 bytes of the returned signature string to determine whether it is the DSPAM ISR or not.  Once found, the DSPAM multiplex number must be used for all future API calls.

Registers upon Entry:	AH	=	Multiplex number (0 to 255)

AL	=	00h

Registers upon Exit:	AL	=	FFh if multiplex number is in use,
00h if free

CX	=	Version number (CH = major, CL = minor)

DX:DI	=	Pointer to signature string (see below) identifying the ISR using the multiplex number

The format of the signature string is as follows:

Offset	Size	Description

00h	8 bytes	blank-padded manufacturer's name (will be 'SIERRA  ')

08h	8 bytes	blank-padded product name (will be 'DSPAM   ')

10h	64 bytes	ASCIIZ product description (optional, may be a single 00h)

Get Entry Point	Function 01h

This function is used to return a direct call entry point address for the ISR.  Some functions of DSPAM require calls to be made to this entry point address.  Other calls may be done using either method.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	01h

Registers upon Exit:	AL	=	FFh (indicates that entry point is supported)

DX:BX	=	Pointer to entry point for bypassing interrupt chain

Uninstall	Function 02h

This function is used to uninstall the driver.  DSPAM does not currently support this function.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	02h

DX:BX	=	Return address for sucessful uninstall (will be ignored)

Registers upon Exit:	AL	=	00h (not implemented)

Request Pop-up	Function 03h

This function is used to pop-up a TSR screen.  DSPAM does not currently support this function.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	03h

Registers upon Exit:	AL	=	00h (not implemented)

Determine Chain	Function 04h

This function is used to determine if the driver is chained on any other interrupt.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	04h

BL	=	Interrupt number to check (except 2Dh)

Registers upon Exit:	AL	=	Status:
	00h = Not implemented
	01h = Unable to determine
	02h = Interrupt hooked
	03h = Interrupt hooked, address returned
	FFh = Interrupt not hooked

DX:BX	=	If AL=03h, pointer to BL interrupt handler

Get Handle Flag	Function 10h

This function returns a bit-mapped flag indicating the currently loaded DSP application handles or available ROM application numbers.  A DSP application handle is an identifier that is assigned when the DSP app is loaded.  A ROM application number identifies a DSP application stored in system ROM.  Up to 15 application handles may be assigned at a given time.

Also see "Query Application ID" and "Query Application Info" for more information.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	10h

BL	=	0 for loaded application handles
1 for available ROM application numbers

Registers upon Exit:	AL	=	Status:
	00h = Handle returned in BX
	FFh = Device driver busy

BX	=	DSP application handle flag:
	Each bit set (0 to 14) indicates a different 
	handle or ROM application number.  (Bit 0
	is handle #0, bit 1 is handle #1, etc.)

Query Application ID	Function 11h

This function returns an ASCIIZ string containing the DSP application ID name corresponding to the requested application handle or ROM application number.  The application ID name corresponds to the ID_NAME string defined in the DSP Configuration File.

Also see "Get Handle Flag," for more information.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	11h

BH	=	0 for loaded application handle ID
1 for ROM application ID

BL	=	Handle ID or ROM app number (0 to 14)

Registers upon Exit:	AL	=	Status:
	00h = ID name obtained
	01h = DSP application handle not found
	02h = No ID name currently available
	FFh = Device driver busy

DX:DI	=	Pointer to application ID name (ASCIIZ string)

Query Application Info	Function 12h

This function returns detailed information regarding a loaded DSP application or a ROM DSP application.

Also see "Get Handle Flag," for more information.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	12h

BH	=	0 for loaded application handle
1 for ROM application number

BL	=	Handle ID or ROM app number (0 to 14)

Registers upon Exit:	AL	=	Status:
	00h = Information obtained
	01h = DSP application handle not found
	02h = No information currently available
	FFh = Device driver busy

CX	=	Length of information structure (in bytes)

DX:BX	=	Pointer to information structure (see below)

The information structure is currently defined as follows:

Offset	Size	Description
00h	WORD	DSP application ID number

02h	30 bytes	DSP application name and description (ASCIIZ string)

20h	WORD	DSP application version number

22h	WORD	DSP application format:

0 = ROM-based application

1 = RAM-based application

24h	WORD	Total number of code/data chunks in this file (1 or 2)

26h	WORD	Encryption key for security,

0 = no encryption

28h	WORD	DSP hardware manufacturer ID:

0 = Sierra Semiconductor Corp.

2Ah	WORD	DSP product ID:

18025, 18026, etc.

2Ch	DWORD	DSP Compatibility bit map, hardware dependent

30h	DWORD	RAM Compatibility bit map, hardware dependent

34h	1 byte	Minimum DSP app utilization, 0 to 100 percent

35h	1 byte	Maximum DSP app utilization, 0 to 100 percent

36h	WORD	Maximum number of synthesizer voices allowed 
		simultaneously

38h	DWORD	Length of DSP data chunk 1 in bytes

3Ch	WORD	Data chunk 1 type:
			1 = Program code chunk

2 = Program data chunk

3 = Combined program code/data chunk

3Eh	DWORD	DSP RAM absolute load address for chunk 1

42h	DWORD	Reserved for future use

46h	DWORD	Reserved for future use

4Ah	DWORD	Length of DSP data chunk 2 in bytes

4Eh	WORD	Data chunk 2 type:
			1 = Program code chunk

2 = Program data chunk

3 = Combined program code/data chunk

50h	DWORD	DSP RAM absolute load address for chunk 2

54h	DWORD	Reserved for future use

58h	DWORD	Reserved for future use

Load DSP Application	Function 13h

This function is responsible for selecting and/or loading a new DSP application.  A DSP application may be loaded three different ways:  Either selected from Aria system ROM, downloaded from the PC by specifying a PC memory address, or downloaded from the PC by specifying a DSP ID string.  

If using a DSP ID string, the driver will first search the configuration file for a matching ID and corresponding DSP file name.  If the ID string is not found in the configuration file, the driver will then search the DSP system ROM.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	13h

BH	=	DSP application load type:
	00h = Aria system ROM application
	01h = Download app from PC memory
	02h = Download app using ID string -- For
		this option, the DSPAM ISR must be 
		called directly using the entry point 
		address returned by "Get Entry Point."

BL	=	Application number (if BH=0 or Aria I system):
	00h = Aria mode (Synthesizer + PCM audio)
	01h = Sound Blaster emulation mode 
		(default)
	05h = Aria Listener
	07h = Reverb
	08h = QSoundÔ Virtual Audio

DX:SI	=	If BH=1, pointer to DSP application loaded in 
		PC memory.
If BH=2, pointer to ASCIIZ ID string.  This ID 
		string must be one of the defined ID
		NAMES in the DSP Configuration 
		File.

Registers upon Exit:	AH	=	Application handle (00h to 0Eh)

AL	=	Status:
	00h = No error - application loaded properly
	01h = DSP application not found or
		illegal app
	02h = DSP overallocated - cannot load app
	03h = DSP application time-out error - 
		cannot load app
	04h = DSP port not responding
	05h = DSP application already loaded
	FFh = Device driver busy

Unload DSP Application	Function 14h

This function is used to unload an application from the DSP.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	14h

BL	=	Application handle number (0 to 14)

Registers upon Exit:	AL	=	Status:
	00h = Application unloaded
	01h = DSP application handle not found
	02h = DSP port not responding
	FFh = Device driver busy

Get DSP Utilization	Function 15h

This function is used to determine the current DSP utilization.  This percentage value is only relative to the type of DSP in use.  DSPAM uses this number to determine if new applications are allowed to load.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	15h

Registers upon Exit:	AL	=	Status:
	00h = Application unloaded
	01h = Cannot determine utilization
	FFh = Device driver busy

BX	=	If AL=0, utilization percentage:
	0 to 10000 = 0.00% to 100.00%

Get System Mode	Function 16h

This function is used to determine the current DSP system mode and the synthesizer playback mode.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	16h

Registers upon Exit:	AL	=	Status:
	00h = System mode returned
	01h = Cannot determine mode
	FFh = Device driver busy

BH	=	If BL=0, synthesizer playback mode:
	00h = 4 wave audio channels, no synthesis
	01h = 2 wave audio channels, 20 op synth
	02h = 2 wave audio channels, 32 op synth

BL	=	System mode:
	00h = Aria mode
	01h = Sound Blaster emulation mode

Set System Mode	Function 17h

This function is used to set the DSP system mode and the synthesizer playback mode.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	17h

BH	=	If BL=0, synthesizer playback mode:
	00h = 4 wave audio channels, no synthesis
	01h = 2 wave audio channels, 20 op synth
	02h = 2 wave audio channels, 32 op synth

BL	=	System mode:
	00h = Aria mode
	01h = Sound Blaster emulation mode

Registers upon Exit:	AL	=	Status:
	00h = System mode set
	01h = Illegal mode specified
	02h = DSP overallocated
	03h = DSP could not load system application
	04h = DSP port not responding
	FFh = Device driver busy

Set Callback	Function 18h

This function is used to register or remove an application callback address.  Several different combinations of callback triggers are possible.  The callback function located at the specified address must be an interrupt service routine (ISR).  It should return with an IRET instruction.

Registers upon Entry:	AH	=	DSPAM multiplex number (0 to 255)

AL	=	18h

BL	=	Action type:
	00h = Remove previously registered address
	01h = Register new callback address

CX	=	If BL=1, Callback type or trigger:
	Bit 0 set = DSP application loaded
	Bit 1 set = DSP application unloaded
	Bit 2 set = Synthesizer mode change

DX:SI	=	Pointer to application callback function (ISR)

Registers upon Exit:	AL	=	Status:
	00h = No error
	01h = Too many callbacks registered, 
		cannot add
	02h = Address not found, cannot remove
	FFh = Device driver busy

The callback function may not call the DSPAM device driver or any DOS interrupt function.  Upon entry into the callback function, the registers will be set as follows:

DX	=	Callback type:

Bit 0 set = DSP application loaded

Bit 1 set = DSP application unloaded

Bit 2 set = Synthesizer mode change

BL	=	DSP application handle loaded or unloaded (if DX bit 0 or 1 set)

BH	=	New synthesizer mode (if DX bit 2 set):

00h = 4 wave audio channels, no synthesis
01h = 2 wave audio channels, 20 op synthesis
02h = 2 wave audio channels, 32 op synthesis

CX	=	Number of Aria Synthesizer voices (operators) allowed

XE "DSP Application Manager" \r "DSPAM"§XE "DOS Application Interface" \r "DOS APPLICATION INTERFACE"§

< This page intentionally left blank >